# Tasks

| Sl No. | Component and required specification | Quantity | Estimated total BDT | Budget USD BDT ÷ 123 | Purpose |
| --- | --- | --- | --- | --- | --- |
| 1 | Raspberry Pi 5 Model B, 8 GB | 1 | 35,600.00 | $289.43 | Runs live command decoding, camera control, LiDAR processing and the results viewer. |
| 2 | Official Raspberry Pi 5 Active Cooler | 1 | 840.00 | $6.83 | Cools the Pi during operation. Keep the airflow path clear. |
| 3 | SanDisk Extreme PRO 64 GB A2 microSDXC, SDSQXCU-064G-GN6MA | 1 | 3,100.00 | $25.20 | Stores the operating system, program, photographs, distance results and command logs. |
| 4 | Official Raspberry Pi 5 RTC battery | 1 | 899.00 | $7.31 | Retains the clock through power-off. Set and verify the date/time before each run. |
| 5 | Raspberry Pi Camera Module 3 Standard; 12 MP autofocus, normal field of view, integrated IR-cut filter | 1 | 6,920.00 | $56.26 | Captures target images and supports visual alignment of the LiDAR. Software applies the requested filter. |
| 6 | Pi 5 Standard–Mini camera FPC cable; 22-pin to 15-pin, approximately 300 mm | 1 | 390.00 | $3.17 | Connects the camera to the Pi. Provide strain relief and a loop for the verified pan travel. |
| 7 | Genuine RTL-SDR Blog V3 USB receiver; RTL2832U with R820T2/R860 tuner, SMA female | 1 | 4,914.00 | $39.95 | Receives NASA’s 144–148 MHz FM signal for live 1200-baud AX.25/APRS decoding. Receive only. |
| 8 | Passive 144–148 MHz receive antenna kit; 50 Ω, matching coax, SMA male plug and insulated mount | 1 kit | 1,500.00 | $12.20 | Receives the command broadcast. Match antenna and cable; keep the assembly within the rover size limit. |
| 9 | Shielded USB-A male to USB-A female extension cable; 30–50 cm | 1 | 500.00 | $4.07 | Allows separation of the SDR from motor wiring, switching converters and other interference sources. |
| 10 | Waveshare 7inch HDMI LCD (C), Rev 4.1; 1024 × 600 capacitive touch, with protective mount | 1 | 5,911.00 | $48.06 | Displays results at PER. Keep combined USB/fan draw within 1.6 A after confirmed 5 A power negotiation; measure with all devices operating. |
| 11 | Display cable kit: micro-HDMI male to HDMI-A male; USB-A male to micro-USB-B data cable | 1 kit | 600.00 | $4.88 | Connects display video, touch and USB power. Count suitable bundled cables toward this kit. |
| 12 | Benewake TF03-100 UART/CAN version, DFRobot SEN0328; 100 m nominal range, IP67, 3.3 V TTL UART, 115200 baud / 8N1 | 1 | 27,047.70 | $219.90 | Measures distance for H commands. Use regulated 5.2 V, aim at the intended board, and record valid results in feet. |
| 13 | CP2102 USB-A to UART adapter; 3.3 V TX/RX logic, Linux support, 115200 baud or faster; TX, RX and GND terminals | 1 | 250.00 | $2.03 | Links LiDAR to the Pi. Cross TX/RX and join GND; leave adapter VCC disconnected. Do not use an RS-232 adapter. |
| 14 | TF03 power/UART cable or mating pigtail; match the supplied sensor revision and expose VCC, GND, TX and RX | 1 | 250.00 | $2.03 | Connects LiDAR power and data. Connector allowance; use a suitable supplied cable instead if included. |
| 15 | Tower Pro MG996R Robot 180° positional servo, with matching metal horn; purchase conditional on required pan travel | 1 | 2,000.00 | $16.26 | Pans the camera/LiDAR platform in calibrated 30° steps. Confirm total travel with NASA before purchase; the handbook does not specify it. |
| 16 | FEETECH FS90 positional micro servo; 4.8–6 V, approximately 120° travel, 9 g, with matching horn; not FS90R | 1 | 1,033.20 | $8.40 | Tilts the LiDAR automatically. Accepts 3.3 V PWM from the shared PCA9685; use its own regulated power branch. |
| 17 | PCA9685 16-channel 12-bit PWM breakout with headers; VCC and I²C pull-ups at 3.3 V | 1 | 470.00 | $3.82 | Controls both servos at a common calibrated servo frequency. Keep separate servo power rails off the shared V+ bus. |
| 18 | SN74AHCT125N buffer kit: IC, 14-pin socket, one 100 nF ceramic capacitor and one 10 kΩ input pull-down | 1 kit | 200.00 | $1.63 | Converts pan-servo PWM from 3.3 V to 5 V. Power from the pan UBEC; common GND; fix unused inputs and enable pins to defined levels. |
| 19 | Shared pan mount: bearing-supported platform, MG996R bracket, camera plate with adjustable fixed elevation, fasteners, hood and cable restraints | 1 kit | 1,000.00 | $8.13 | Supports the camera and LiDAR tilt assembly. Revised fabrication allowance; verify payload, balance, backlash and full cable clearance. |
| 20 | TF03 tilt cradle, FS90 bracket and pan-platform adapter; balanced bearing-supported pivot, revision-matched sensor mounting hardware | 1 kit | 1,000.00 | $8.13 | Provides LiDAR elevation adjustment. Fabrication allowance; calibrate beam alignment and verify that the beam hits the intended board. |
| 21 | Dedicated task LiPo battery: 3S, 11.1 V nominal, 2200 mAh, ≥25C continuous; 12.6 V fully charged; XT60 main connector and 4-pin JST-XH balance lead | 1 | 2,450.00 | $19.92 | Powers task electronics only; 24.42 Wh nominal. Local budget estimate; verify brand, connectors and pack condition. Measure runtime through results review; keep separate from the drive battery. |
| 22 | Separate insulated battery mounting kit: rigid tray/enclosure, cushioning, retaining straps, fasteners and protected cable exits; fit the supplied 2200 mAh pack | 1 kit | 650.00 | $5.28 | Secures and electrically isolates the battery, separately from other components as required by the handbook. Fabrication allowance; include it in rover mass and size checks. |
| 23 | Task battery main connection/protection kit: XT60 mating connector, short 14 AWG leads, covered 7.5 A fuse and holder (≥32 V DC), retained master switch rated ≥10 A at 24 V DC | 1 kit | 750.00 | $6.10 | Main fuse close to battery positive, then switch, guard and branch distribution. Extra to the four branch fuses. Allowance; confirm 7.5 A selection with measured load/inrush and component ratings. |
| 24 | Kemo M148A battery guard: adjustable 10.4–13.3 V cutoff, 10 A without added cooling; switches the negative return. Set and bench-verify approximately 10.5 V cutoff | 1 | 3,500.00 | $28.46 | Delayed pack-level cutoff; not individual-cell protection. All task returns must pass through its load side: no chassis/USB bypass. Auto-reconnects about 0.8 V above cutoff; shut down and switch off after an alarm. Import budget allowance. |
| 25 | Adjustable per-cell LiPo voltage alarm/tester: supports 3S via 4-pin JST-XH balance connector; individual-cell display and audible alarm; set approximately 3.6 V/cell | 1 | 220.00 | $1.79 | Early low-cell warning; does not disconnect power. Verify threshold against pack guidance. Connect only to the balance lead, insulated from task ground; unplug after use. Stop before continued discharge. |
| 26 | 52Pi EP-0225 Pi 5 PD power board; 9–24 V DC input, USB-PD 5.15 V / 5 A output; supplied 5 A-capable USB-C cable and mounting hardware | 1 kit | 2,822.00 | $22.94 | Powers the Pi from the protected dedicated task battery. Confirm 5 A PD negotiation and input above 9 V during peak load. Use only one input. |
| 27 | Center-positive DC input pigtail; exact plug and current rating matched to the supplied EP-0225 board | 1 | 150.00 | $1.22 | Connects the protected task supply to the Pi power board. Verify polarity and fit before energizing. |
| 28 | Genuine Hobbywing UBEC 3A; 2–6S input, output set to 5 V | 1 | 1,106.00 | $8.99 | Powers the MG996R and its PWM buffer separately from the Pi. Verify voltage during pan movement and starting load. |
| 29 | DFRobot DFR0379 adjustable DC-DC buck converter; 4–40 V input, 3 A maximum; ≤2 A recommended without extra cooling; set output to 5.2 V | 2 | 1,205.40 | $9.80 | One for LiDAR, one for FS90. Meter before connection; keep LiDAR at 5.0–5.5 V under load. Separate positive outputs; common protected task GND. |
| 30 | 1000 µF, 25 V low-ESR electrolytic capacitor; e.g. Panasonic EEU-FR1E102E | 3 | 396.00 | $3.22 | One at each load: pan servo, LiDAR and tilt servo. Observe polarity; check start-up and transient stability. |
| 31 | 100 µF, 25 V electrolytic capacitor for converter input decoupling | 2 | 40.00 | $0.33 | One across each DFR0379 input, close to the converter. Budget allowance. |
| 32 | 100 nF, ≥50 V X7R ceramic capacitor | 4 | 40.00 | $0.33 | Two converter-input bypasses and two load bypasses. The separate buffer kit includes its own fifth 100 nF capacitor. |
| 33 | Task branch harness: 4 covered fuse holders; 1 × 5 A, 1 × 3 A and 2 × 1 A time-delay fuses, all ≥32 V DC; short 14 AWG distribution feed, 18 AWG branches, terminals and heat-shrink | 1 kit | 1,600.00 | $13.01 | Pi input: 5 A; pan UBEC: 3 A; each DFR0379: 1 A. Feed from the task master switch and battery guard. Allowance; verify fuse, wire and inrush ratings. |
| 34 | Vented task enclosure and signal-wiring kit: standoffs, perfboard, glands, locking connectors, Pi I²C leads, UART leads and two servo signal/power leads | 1 kit | 1,250.00 | $10.16 | All task grounds join on the battery guard’s load side. Keep them isolated from raw battery negative, chassis and drive-system grounds. Separate regulated positive rails. Allowance. |
| 35 | Existing 2–6S LiPo balance charger, matching PSU and charging/storage bag; compatible XT60 charge lead and 3S JST-XH balance adapter | 1 set | 250.00 | $2.03 | Reuse the charger/bag already budgeted for the rover. Allowance only for missing leads. Off-rover 3S LiPo balance charge: 4.2 V/cell, ≤2.2 A or the pack’s lower limit. Disconnect rover electronics for charging. |
| 36 | Software: Raspberry Pi OS 64-bit; compatible rtl-sdr/rtl_fm, ALSA audio routing, Dire Wolf, Picamera2, OpenCV, Python, pyserial and PCA9685 I²C driver | 1 set | 0.00 | $0.00 | Free software foundation for reception, camera, servo and LiDAR interfaces. Install compatible versions and test the complete receiver chain. |
| 37 | Team-developed task controller and results viewer: live AAA/B/C/D/E/F/G/H/ZZZ parser, pan/aim calibration, filter state, timestamped image recorder and LiDAR logger | 1 app | 0.00 | $0.00 | Develop/test autonomous commands, calibrated aiming, replacing filters (G = normal colour), timestamped images and fresh valid H readings (cm ÷ 30.48). Restore commanded view; save each result promptly for PER; provide safe shutdown. |
|  | TOTAL |  | 110,854.30 | $901.25 | Planning estimates; obtain supplier quotes. Excludes shipping, taxes and paid labour. Reused charger/bag not repurchased. USD total converted before rounding. |